OwnTech SPIN / TWIST Timing Benchmark

Experiment Setup Guide :- wiring, build settings, procedure E0–E7, data recording, complete source code

Companion to: OwnTech_TWIST_Timing_Program_EN.docx and main_timing_EN.cpp .

Target: SPIN 1.2.0 (STM32G474RE, Cortex-M4F, 170 MHz) with TWIST 1.4.2 shield, OwnTech Core on Zephyr RTOS, PlatformIO/VS Code. Safety. Experiments E0–E6 run with the TWIST power stage unpowered: USB only. No PWM, no power-stage enable and no ADC acquisition is started by the benchmark. E7 (full control loop with PWM) is a separate experiment and follows the OwnTech power-up procedure of your lab.

1. Review 	1

2. Equipment	1

3. Pin selection and wiring	1

3.1 SPIN header pin → STM32 pin map	1

3.2 Oscilloscope wiring	1

4. Software settings	1

4.1 Project files	1

4.2 Compile-time switches in main.cpp	1

4.3 Console	1

5. What the program measures	1

5.1 Equations	1

6. Oscilloscope settings	1

7. Procedure	1

7.1 Preparation (once)	1

7.2 E0 — instrumentation calibration (mode 0)	1

7.3 E0b — clock validation (mode 2)	1

7.4 E1–E5 — bubble-sort sweep (mode 1)	1

7.5 E6 — MMC algorithm in the target window	1

7.6 E7 — full control loop	1

8. Data recording	1

8.1 CSV line format	1

8.2 DWT results (fill in)	1

8.3 Oscilloscope cross-validation (fill in)	1

8.4 Configuration control (fill in once per campaign)	1

8.5 Acceptance criteria	1

9. Troubleshooting	1

10. Complete source code — main.cpp (v2)	1

11. References	1

1. Review

| Item | Verified against (OwnTech Core / ARM) |
| --- | --- |
| spin.gpio.configurePin/setPin/resetPin with pin 9 | GpioHAL.h/.cpp: pin is uint8_t; 9 → GPIOC bit 7 (PC7); same pin as OwnTech's own GPIO example; power-leg.yaml table confirms PC7 = SPIN 9 |
| task.createCritical(fn, 100, source_tim6) | TaskAPI.h: period 1…6553 µs, returns 0/-1; uninterruptible_synchronous_task.cpp: TIM6 path |
| task.startCritical(false) | scheduling_start_uninterruptible_synchronous_task(): with TIM6 and the default argument (true) it divides by the HRTIM period, which is 0 when PWM is not initialised, and returns WITHOUT starting the timer |
| Execution context of the critical task | stm32_timer_driver.c: TIM6 UPDATE IRQ registered with IRQ_ZERO_LATENCY → the task is an ISR above the kernel's priority mask |
| DWT_CYCCNT register map | ARM DDI0439 / ST PM0214 |
| CPU_FREQ_HZ = 170 MHz hard-coded | spin.dts: rcc clock-frequency = 170 MHz; the TIM6 driver computes its prescaler from CONFIG_SYS_CLOCK_HW_CYCLES_PER_SEC |
| GPIO edges through the Zephyr GPIO API Works; ~100–200 cycles per edge, asymmetric around the DWT window | GpioHAL.cpp → if/else pin lookup → gpio_pin_set() (with CONFIG_ASSERT=y) |
| Interrupt-entry latency | TIM6 counts 0.1 µs ticks from 0 at each event |
| Actual period / jitter / overruns | — |
| Machine-readable output | — |
| g_snapshot.batch_id++ on a volatile | GCC |
| sum_cycles as uint32 Fine at Ts = 100 µs (overflow only beyond 25 ms per sample) | — |
| Unused static functions in mode 0 | — |
| Pin conflict risk | hrtim.dtsi: PC7 is HRTIM TIMF2, muxed only when PWMF is initialised; spin.dts: TIM3 encoder pinctrl (PC6/PC7) applied only if the encoder is started |

What did not change: the experimental logic (E0 calibration → bubble sort sweep → MMC), the batch/snapshot architecture, the two-method cross-validation and the acceptance criteria of the v1 document remain valid. Use the v1 document for the method description and this guide for execution.

# 2. Equipment

| Item | Requirement | Why |
| --- | --- | --- |
| OwnTech SPIN 1.2.0 | Programmed through USB-C; TWIST 1.4.2 shield may stay mounted | Target MCU STM32G474RE at 170 MHz |
| USB-C cable to PC | Data-capable cable | Power, firmware upload, CDC-ACM console (115200) |
| Oscilloscope | ≥ 50 MHz analogue bandwidth; pulse-width (+Width) measurement with Min/Mean/Max statistics; infinite persistence useful | External measurement of the target duration (method B) |
| Probe | 10× passive probe, short ground spring/lead | 3.3 V logic edges; keep ground path short to avoid ringing |
| Connection to pin 9 | 2.54 mm header pin or probe hook on SPIN header pin 9, plus a GND pin | Benchmark pulse output (PC7) |
| PC software | VS Code + PlatformIO + an OwnTech Core project that already builds and uploads an official example | Toolchain (Zephyr, arm-none-eabi-gcc) comes with the OwnTech project |
| Optional: ST-LINK | Only for debugging; not needed for the benchmark | DWT works without a debugger attached (TRCENA is set by software) |

# 3. Pin selection and wiring

## 3.1 SPIN header pin → STM32 pin map

The table is a transcription of GpioHAL::getPinNumber() and GpioHAL::getGpioDevice() (OwnTech Core). Only the 45 GPIO-capable header pins appear; the remaining header positions (3, 8, 13, 18, 23, 28, 33, 36, 38–40, 54, 57) are not GPIO in the API (power, ground or reserved — check the SPIN 1.2.0 pinout figure for the GND positions). The “use” column lists what the SPIN device tree or the TWIST 1.4.2 overlay assigns to the pin; “—” means neither file references it.

| SPIN pin | STM32 | Use in spin.dts / TWIST 1.4.2 overlay / hrtim.dtsi | Probe candidate |
| --- | --- | --- | --- |
| 1 | PB11 | — | yes |
| 2 | PB12 | TWIST LEG2 PWM high (HRTIM PWMC1) | no |
| 4 | PB13 | TWIST LEG2 PWM low (HRTIM PWMC2) | no |
| 5 | PB14 | HRTIM PWMD1 (unused by TWIST) | possible |
| 6 | PB15 | HRTIM PWMD2 (unused by TWIST) | possible |
| 7 | PC6 | TWIST LEG1 capacitor pin; HRTIM PWMF1; TIM3 CH1 (encoder) | no |
| 9 | PC7 | HRTIM PWMF2 (unused by TWIST); TIM3 CH2 (only if encoder started). OwnTech GPIO example pin. | RECOMMENDED |
| 10 | PC8 | HRTIM PWME1 (unused by TWIST) | alternate |
| 11 | PC9 | HRTIM PWME2 (unused by TWIST) | alternate |
| 12 | PA8 | TWIST LEG1 PWM high (HRTIM PWMA1) | no |
| 14 | PA9 | TWIST LEG1 PWM low (HRTIM PWMA2) | no |
| 15 | PA10 | HRTIM PWMB1 (unused by TWIST) | possible |
| 16 | PC10 | USART3 TX (RS485 on TWIST) | no |
| 17 | PC11 | USART3 RX (RS485 on TWIST) | no |
| 19 | PC12 | TWIST LEG1 driver enable | no |
| 20 | PB4 | — | yes |
| 21 | PB9 | — | yes |
| 22 | PC13 | TWIST LEG2 driver enable | no |
| 24 | PC0 | TWIST sensor input (ADC) | no |
| 25 | PC1 | TWIST LEG2 current-mode / sensor input (ADC) | no |
| 26 | PC2 | — (ADC-capable) | yes |
| 27 | PC3 | TWIST sensor input (ADC) | no |
| 29 | PA0 | TWIST sensor input (ADC) | no |
| 30 | PA1 | TWIST LEG1 current-mode / sensor input (ADC) | no |
| 31 | PB0 | — | yes |
| 32 | PA5 | SPIN user LED | no |
| 34 | PA6 | — | yes |
| 35 | PC4 | — | yes |
| 37 | PB1 | HRTIM SCOUT — TWIST sync output | no |
| 41 | PB10 | — | yes |
| 42 | PB2 | HRTIM SCIN — TWIST sync input | no |
| 43 | PC5 | — | yes |
| 44 | PA7 | — | yes |
| 45 | PA4 | — | yes |
| 46 | PA13 | SWDIO (debug) | NEVER |
| 47 | PA14 | SWCLK (debug) | NEVER |
| 48 | PA15 | — | yes |
| 49 | PD2 | TIM3 ETR (encoder, only if started) | possible |
| 50 | PB3 | TIM4 ETR (encoder, only if started) | possible |
| 51 | PA2 | LPUART1 TX (SPIN) | no |
| 52 | PA3 | LPUART1 RX (SPIN) | no |
| 53 | PB5 | FDCAN2 RX (TWIST) | no |
| 55 | PB6 | FDCAN2 TX (TWIST); TIM4 CH1 | no |
| 56 | PB7 | TWIST LEG2 capacitor pin; TIM4 CH2 | no |
| 58 | PB8 | — | yes |

Why pin 9 is safe. PC7 is the second output of HRTIM unit F. OwnTech muxes HRTIM pins to the timer only inside hrtim_tu_gpio_init(), i.e. when spin.pwm.initUnit(PWMF) (or the shield power API for a leg that uses PWMF) is called. TWIST legs use PWMA (LEG1) and PWMC (LEG2), so PC7 remains a plain GPIO even in E7. The TIM3 incremental-encoder pinctrl named incremental_encoder on PC6/PC7 is applied only if you start the encoder — do not use the TIM3 encoder API while benchmarking.

If you change TIMING_GPIO_PIN, the code resolves the port and bit for you (also for STM32-style names such as PC8) and rejects non-GPIO pins and the SWD pins at compile time.

## 3.2 Oscilloscope wiring

SPIN 1.2.0 (+ TWIST 1.4.2, power stage OFF)

PC  <==== USB-C =====>  [USB-C]  power + firmware upload + CDC-ACM console 115200

|

Oscilloscope CH1                 |  header pin 9  (STM32 PC7)  o----> probe TIP

10x probe, DC, 1 Mohm            |  header GND pin             o----> probe GROUND clip

|

Trigger: CH1 rising edge ~1.6 V  |  Nothing else connected. No DC bus. No load.

- Probe ground goes to a SPIN GND header pin (logic ground, the same ground as the USB connector). Never to a power-stage node.

- Keep the ground lead as short as possible (ground spring if available); the pulse edges are ~1–2 ns on a 3.3 V output and will ring with a long lead, which perturbs the +Width measurement.

- Use 10× attenuation and DC coupling; set the channel to 1 V/div so 3.3 V fills about a third of the screen.

- The pulse repeats every Ts = 100 µs (10.00 kHz). If you see a different repetition rate, the configured period or the trigger source is not what you think — stop and check.

- For E7 (power stage energised) follow the lab's isolation rules; the SPIN logic ground reference of the TWIST may not be at earth potential depending on the converter configuration. Use a differential or isolated probe if in doubt.

# 4. Software settings

## 4.1 Project files

| File | Setting | Value / action |
| --- | --- | --- |
| src/main.cpp | Program | Replace with main.cpp v2 (Section 10). Back up the previous file first. |
| platformio.ini | board / board_version | spin / 1_2_0 (default in the Core project; set to your SPIN revision) |
| platformio.ini | board_shield / board_shield_version | twist / 1_4_2 — keep for E6/E7. For E0–E5 you may comment both lines out to build a bare SPIN image; then record that choice, because it changes which OwnTech modules are compiled. |
| platformio.ini | monitor_speed | 115200 (the console is USB CDC-ACM; the number must match the Serial Monitor setting) |
| platformio.ini | build_flags | -std=c++2a -fsingle-precision-constant (OwnTech default; do not add -O flags here — optimisation is a Zephyr Kconfig choice, see 4.2) |
| zephyr/prj.conf | Optimisation level | Default is size optimisation (-Os). For control code you may add CONFIG_SPEED_OPTIMIZATIONS=y (-O2). Whatever you choose, keep it identical for E0…E7 and record it. |
| zephyr/prj.conf | Nothing else required | CONFIG_PRINTK, CONFIG_FPU, zero-latency IRQs and asynchronous tasks are already enabled by the OwnTech defaults. |

How to see which optimisation level was used: after a build, open .pio/build/<env>/zephyr/.config and look for CONFIG_SIZE_OPTIMIZATIONS=y or CONFIG_SPEED_OPTIMIZATIONS=y. Write the value in the configuration-control row of the result tables.

## 4.2 Compile-time switches in main.cpp

| Macro | Default | Meaning / when to change |
| --- | --- | --- |
| CONTROL_PERIOD_US | 100 | Ts. 1…6553 µs with TIM6 (checked at compile time). Increase to 200 for E5 if N = 64 overruns. |
| TIMING_IRQ_SOURCE_TIM6 | 1 | 1 = TIM6 trigger (E0–E6). 0 = HRTIM master trigger for E7; then PWM must be initialised before createCritical() and Ts must be a multiple of the PWM period. |
| TIMING_GPIO_PIN | 9 | Oscilloscope pin: SPIN number (1…58) or STM32 name (PC8…). Port/bit resolved and checked at compile time. |
| BATCH_SAMPLES | 1000 | Executions per reported batch (0.1 s at Ts = 100 µs). 10000 gives one report per second with a better per-batch maximum. |
| MAX_SORT_N / TEST_SORT_N | 64 / 16 | Bubble-sort size. Sweep TEST_SORT_N = 4, 8, 16, 32, 64 (E1–E5). |
| REFERENCE_DELAY_US | 10 | Length of the DWT-timed busy-wait in mode 2 (clock validation). |
| TIMING_TEST_MODE | 1 | 0 = empty window (E0), 1 = bubble sort (E1–E5), 2 = reference delay (E0b). |
| TIMING_USE_STM32_LL | 1 | 1 = direct GPIO register writes (≈1 cycle per edge) and TIM6->CNT latency read. 0 = OwnTech GPIO API only (portable; set 0 if <stm32_ll_gpio.h> is not found by your build). |
| PRINT_CSV | 1 | Print one CSV line per batch for copy-paste into a spreadsheet. |

Rules for the whole campaign: one firmware image per experiment; change only the macro the experiment calls for; never change optimisation level, shield selection, Ts or TIMING_USE_STM32_LL between E0 and the experiment you compare it with. The batch header line prints Ts, f_CPU, pin and LL on every report so a log file is self-describing.

## 4.3 Console

- The console is the USB virtual COM port of the SPIN (zephyr,console = &cdc_acm_uart0), not the ST-LINK UART. Open the PlatformIO Serial Monitor (115200). The OwnTech default CONFIG_BOOT_DELAY=1500 gives 1.5 s to attach; if the boot banner is missed, every batch report repeats the configuration line.

- Let the monitor log to a file (PlatformIO: monitor_filters / log2file, or any terminal with logging) — the CSV lines are what you paste into the result tables.

- The reporter prints only from the background thread; the critical task never prints (it is a zero-latency ISR — a printk there would be a fault or a corrupted console).

# 5. What the program measures

TIM6 UPDATE event, t = 0 (TIM6->CNT restarts from 0, ticks of 0.1 us)

|

|  L  = entry latency: NVIC entry + Zephyr ISR wrapper + timer_stm32_callback()

|       + user_task_proxy()  -> measured as TIM6->CNT at the first instruction

v

loop_critical_task()

|- read TIM6->CNT                       = L                      -+

|- DWT critical_start                                             |

|- prepare_sort_input()        (mode 1, outside the target)      |

|- GPIO HIGH  ---------------------------------+                  |  C = critical body

|- DWT target_start                            |                  |

|- task_under_test()                 = T       |  P = scope pulse |

|- DWT target_end                              |                  |

|- GPIO LOW   ---------------------------------+                  |

|- result guard                 (mode 1)                          |

|- DWT critical_end                                              -+

'- statistics + snapshot        (not measured, ~50-100 cycles)

ISR exit -> background thread resumes ... next TIM6 event after Ts   (period = entry-to-entry)

| Quantity | Symbol | How | Resolution |
| --- | --- | --- | --- |
| Target duration | T | DWT_CYCCNT difference around task_under_test() | 1 cycle = 5.882 ns |
| Critical body | C | DWT_CYCCNT from first to last instruction of the callback (excludes statistics) | 1 cycle |
| Entry latency | L | TIM6->CNT at callback entry × 17 cycles per tick (TIM6 source only) | 0.1 µs = 17 cycles |
| Period | — | DWT critical_start[n] − critical_start[n−1]; jitter = max − min | 1 cycle |
| Overruns | — | Count of executions with C ≥ Ts (per batch and since boot) | — |
| Scope pulse | P | +Width of the HIGH pulse on pin 9; P = T + δ where δ is the E0 offset | scope-dependent |

## 5.1 Equations

t [us]        = cycles / f_CPU[MHz]            (170 MHz: 1 cycle = 5.882 ns, Ts = 100 us = 17 000 cycles)

Worst case    = L_max + C_max                  (ISR exit path, ~20-40 cycles, is not included)

Slack         = Ts - (L_max + C_max)

Utilisation   = (L_max + C_max) / Ts * 100 %

Scope offset  : delta = P_E0 - T_E0            (from E0, mode 0)

Cross-check   : error % = | (P - delta) - T | / T * 100   (per experiment, using max or mean consistently)

Clock check   : f_actual = f_assumed * T_DWT,E0b / (P_E0b - delta)   (E0b, mode 2; expect 170.0 MHz +/- crystal tolerance)

Deadline verdict printed by the firmware: PASS when no overrun occurred in the batch and L_max + C_max < Ts; otherwise FAIL with the overrun counts. A FAIL is a valid experimental result (e.g. N = 64 at Ts = 100 µs) — the firmware keeps running and the scope still shows the pulse.

Terminology: all maxima are observed maxima over the samples collected, not a formal WCET. Say “measured maximum over n samples” in the thesis unless a static analysis is added.

# 6. Oscilloscope settings

| Setting | Value | Comment |
| --- | --- | --- |
| CH1 | 1 V/div, DC, 10× probe | 3.3 V logic |
| Trigger | CH1, rising edge, level ≈ 1.6 V, mode Normal | One trigger per Ts; Normal mode so nothing is drawn without a pulse |
| Timebase E0 | 100–200 ns/div | Pulse ≈ tens of ns with LL = 1; ≈ 1–2 µs with LL = 0 |
| Timebase E0b | 2 µs/div | Pulse ≈ 10 µs + δ |
| Timebase E1–E5 | start at 5 µs/div, then adapt | Pulse grows ≈ ×4 per doubling of N |
| Measurement | +Width on CH1 with statistics (Min / Mean / Max / count) | Reset statistics at the start of each experiment; collect ≥ 10 s (≥ 100 000 pulses) |
| Acquisition | Normal (no averaging), highest sample rate, infinite persistence on | Averaging would hide the maximum; persistence shows the jitter envelope |
| Sanity check | Frequency / period measurement on CH1 | Must read 10.00 kHz / 100.0 µs for Ts = 100 µs |

# 7. Procedure

## 7.1 Preparation (once)

- Open the OwnTech project that already builds and uploads an official example. Build it once to be sure the toolchain works.

- Back up src/main.cpp; copy main.cpp v2 in its place.

- Check platformio.ini: board_version matches your SPIN; shield lines as decided; monitor_speed = 115200. Decide the optimisation level (4.1) and do not touch it again.

- Wire the probe (3.2). Power the board through USB only.

- Build with TIMING_TEST_MODE 0 and upload. Open the Serial Monitor. You must see the banner with “DWT cycle counter: OK”, the CSV_HEADER line, and then a batch report every 100 ms. If you see “ERROR: critical task not started”, read Section 9.

## 7.2 E0 — instrumentation calibration (mode 0)

- Firmware: TIMING_TEST_MODE = 0, TIMING_USE_STM32_LL = 1 (and optionally repeat with 0 to quantify the OwnTech GPIO API path).

- Record from the console: Target min/avg/max (expected: a few cycles — the empty noinline call plus two DWT reads), Critical body (expected ≈ 10–30 cycles), Entry latency (expected roughly 0.3–1.0 µs; quantised to 0.1 µs), Period min/avg/max (expected 17 000 ± a few tens of cycles), overruns = 0.

- Record from the scope: P_E0 Min/Mean/Max. Compute δ = P_E0,mean − T_E0,mean. With LL = 1 expect δ in the tens of nanoseconds; with LL = 0 expect ≈ 1–2 µs.

- If the entry latency is much larger than 1 µs or the period jitter is more than a few hundred cycles with the board idle, something else is running (USB console traffic is expected to add some jitter; CAN/RS485 or encoder activity should not be present).

## 7.3 E0b — clock validation (mode 2)

- Firmware: TIMING_TEST_MODE = 2, REFERENCE_DELAY_US = 10.

- DWT will report T ≈ 1700 cycles (+ loop exit, a few cycles) by construction. The oscilloscope is the independent reference: expect P_E0b = 10.00 µs + δ.

- Compute f_actual with the equation in 5.1. If P − δ = 10.6 µs the core runs at 160 MHz, not 170 MHz, and every DWT-derived time in the campaign must be rescaled. A result within ±0.1 % validates CPU_FREQ_HZ (the HSE crystal tolerance dominates).

## 7.4 E1–E5 — bubble-sort sweep (mode 1)

- Firmware: TIMING_TEST_MODE = 1; one build per N = 4, 8, 16, 32, 64 (TEST_SORT_N). Everything else fixed.

- For each N: wait for at least 10 batches, copy 3–5 CSV lines, take the largest t_max and c_max among them; reset the scope statistics, collect ≥ 10 s, record P Min/Mean/Max.

- Expected shape: T grows ≈ N(N−1)/2 × (cycles per compare-and-swap). Doubling N should multiply T by ≈ 4. At -Os, N = 16 is of the order of 1–2 k cycles (6–12 µs) and N = 64 of the order of 20–30 k cycles (120–180 µs), i.e. N = 64 is expected to overrun Ts = 100 µs. If it does, that is the result for E5 at 100 µs; add an E5′ with CONTROL_PERIOD_US = 200 and note it.

- Fill the DWT table (8.2) and the cross-validation table (8.3). The error between (P − δ) and T should be well below 1 % for N ≥ 16; for E1 (N = 4, pulse of a few hundred ns) expect a larger relative error — the scope resolution dominates there.

## 7.5 E6 — MMC algorithm in the target window

- Replace the body of task_under_test() by a call to the real function (e.g. the local sorting / capacitor-voltage balancing step). Keep input preparation outside the window, as prepare_sort_input() does; keep a visible side effect after the window (like g_result_guard) so the optimiser cannot delete the computation.

- Replace the worst-case input generator by inputs that are hard for your algorithm (reverse-sorted voltages, all-equal voltages, alternating patterns) and run each pattern as a separate sub-experiment; report the largest maximum.

- Keep TIM6 as source and startCritical(false): no ADC, no PWM. Pin 9 stays valid.

## 7.6 E7 — full control loop

- Set TIMING_IRQ_SOURCE_TIM6 0 if the task must be synchronised with the PWM; initialise the PWM units (and ADC) before task.createCritical(); Ts must be an integer multiple of the PWM period; call task.startCritical(true) if the OwnTech data dispatch must feed the ADC values.

- Entry latency is then reported as n/a (TIM6 is not the trigger). T, C, period and overruns remain valid. Widen the DWT/GPIO window to the whole control path (acquisition read → control → sorting → communication → PWM update).

- Pin 9 (PC7 = HRTIM PWMF2) remains free as long as PWMF is not initialised — TWIST uses PWMA and PWMC. If your E7 code uses PWMF, move the timing pin (e.g. SPIN 26 = PC2, 31 = PB0).

# 8. Data recording

## 8.1 CSV line format

One line per batch: CSV,batch,mode,N,samples,t_min,t_avg,t_max,c_min,c_avg,c_max,l_min,l_avg,l_max,p_min,p_avg,p_max,overruns,budget_cycles. All durations are in CPU cycles; divide by 170 to get µs. The header line CSV_HEADER,... is printed once at boot.

| Column | Meaning |
| --- | --- |
| batch | Batch counter since boot |
| mode, N | TIMING_TEST_MODE and TEST_SORT_N of the firmware |
| samples | Executions in the batch (= BATCH_SAMPLES) |
| t_min, t_avg, t_max | Target T (task_under_test only), cycles |
| c_min, c_avg, c_max | Critical body C, cycles |
| l_min, l_avg, l_max | Entry latency L, cycles (0 when not measured) |
| p_min, p_avg, p_max | Entry-to-entry period, cycles (nominal 17 000) |
| overruns | Executions with C ≥ Ts in this batch |
| budget_cycles | Ts in cycles (17 000 at 100 µs / 170 MHz) |

## 8.2 DWT results (fill in)

| Exp. | Mode / N | Samples | T max [cyc] | C max [cyc] | L max [cyc] | Period jitter [cyc] | Overruns | L+C max [µs] | Slack [µs] | PASS/FAIL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E0 | 0 / – | 1000 |  |  |  |  |  |  |  |  |
| E0b | 2 / 10 µs | 1000 |  |  |  |  |  |  |  |  |
| E1 | 1 / 4 | 1000 |  |  |  |  |  |  |  |  |
| E2 | 1 / 8 | 1000 |  |  |  |  |  |  |  |  |
| E3 | 1 / 16 | 1000 |  |  |  |  |  |  |  |  |
| E4 | 1 / 32 | 1000 |  |  |  |  |  |  |  |  |
| E5 | 1 / 64 | 1000 |  |  |  |  |  |  |  |  |
| E6 | MMC |  |  |  |  |  |  |  |  |  |
| E7 | Full loop |  |  |  |  |  |  |  |  |  |

## 8.3 Oscilloscope cross-validation (fill in)

| Exp. | N | T_DWT mean [µs] | T_DWT max [µs] | P mean [µs] | P max [µs] | δ (E0) [µs] | P−δ mean [µs] | Error % (mean) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E0 | – |  |  |  |  | — | — | — | δ = P_E0 − T_E0 |
| E0b | 10 µs |  |  |  |  |  |  |  | f_actual = … |
| E1 | 4 |  |  |  |  |  |  |  |  |
| E2 | 8 |  |  |  |  |  |  |  |  |
| E3 | 16 |  |  |  |  |  |  |  |  |
| E4 | 32 |  |  |  |  |  |  |  |  |
| E5 | 64 |  |  |  |  |  |  |  |  |

## 8.4 Configuration control (fill in once per campaign)

| Item | Value |
| --- | --- |
| Date / operator |  |
| SPIN revision / TWIST revision (or “no shield”) |  |
| OwnTech Core commit or release |  |
| Optimisation (CONFIG_SIZE_OPTIMIZATIONS or CONFIG_SPEED_OPTIMIZATIONS) |  |
| Ts (CONTROL_PERIOD_US), IRQ source |  |
| TIMING_USE_STM32_LL, TIMING_GPIO_PIN |  |
| f_CPU printed at boot / f_actual from E0b |  |
| Oscilloscope model, probe, bandwidth |  |

## 8.5 Acceptance criteria

- Deadline: no overrun and L_max + C_max < Ts in every reported batch of the experiment.

- Slack: positive with the design margin you set (e.g. ≥ 30 % of Ts for the MMC task).

- Repeatability: T_max does not drift between batches by more than the entry-latency quantum plus a few cycles once the pattern is fixed.

- Cross-validation: |(P − δ) − T| / T ≤ 1 % for T ≥ 5 µs; for shorter pulses report the absolute difference instead.

- Clock: f_actual from E0b within ±0.1 % of 170 MHz; otherwise rescale all DWT values and investigate the clock tree.

- Configuration control: table 8.4 completely filled; one firmware image per experiment.

# 9. Troubleshooting

| Symptom | Likely cause | What to do |
| --- | --- | --- |
| No console output at all | Wrong COM port (SPIN enumerates as a USB CDC device, not the ST-LINK port); monitor speed ≠ 115200; cable without data lines | Select the port named “SPIN”/OwnTech; set 115200; try another cable. |
| Banner missing, batches present | Serial Monitor attached after the 1.5 s boot delay | Normal — the batch header repeats Ts, f_CPU, pin and LL. |
| “ERROR: critical task not started (create=-1, dwt=1)” | Period outside 1…6553 µs, or a critical task already defined and running (second createCritical) | Check CONTROL_PERIOD_US; make sure no other code created a critical task. |
| “ERROR: critical task not started (create=0, dwt=0)” | DWT not counting | Should not happen on STM32G474. Power-cycle; check that no debugger script disabled TRCENA. |
| Banner OK, no batch reports | Task defined but not running: startCritical(true) used with TIM6 and no PWM (returns silently); or background task not created | Keep task.startCritical(false) for E0–E6; check the “could not create background task” message. |
| No pulse on the scope, console fine | Probe on the wrong header pin; PC7 claimed by PWMF or by the TIM3 encoder; probe ground open | Verify the “Timing pin: SPIN 9 -> PC7” line; confirm no PWMF/encoder init; check ground clip. |
| Pulse much longer than T_DWT (≈ 1–2 µs offset) | TIMING_USE_STM32_LL = 0: Zephyr GPIO API latency with CONFIG_ASSERT=y | Expected for the API path; subtract δ from E0, or use LL = 1. |
| Pulse rate ≠ 10 kHz | Different Ts in the firmware; HRTIM source with another repetition | Compare with the Ts printed in the batch header. |
| Period jitter of several µs, latency ≫ 1 µs | Another zero-latency or high-priority interrupt is active (RS485 RX, CAN, ADC DMA) | Disable communication modules for the benchmark; record what is enabled. |
| Build error: stm32_ll_gpio.h / stm32_ll_tim.h not found | LL include path not exposed in your Core version | Set TIMING_USE_STM32_LL = 0 (portable path; entry latency becomes n/a). |
| Build error: static_assert “not a GPIO-capable SPIN pin” | TIMING_GPIO_PIN is a power/GND/reserved position or a typo | Use a pin from table 3.1. |
| Deadline FAIL with overruns > 0 | Task longer than Ts (expected for N = 64 at 100 µs) | Valid result. For the sweep add a run with CONTROL_PERIOD_US = 200; for the MMC task reduce the work or lengthen Ts. |
| Period avg ≠ 17 000 while overruns = 0 | f_CPU ≠ 170 MHz or TIM6 prescaler assumption broken | Run E0b; check CONFIG_SYS_CLOCK_HW_CYCLES_PER_SEC in .config. |

# 10. Complete source code — main.cpp (v2)

Drop-in replacement for src/main.cpp. Compiled warning-free on host with GCC -std=c++20 -Wall -Wextra -Wpedantic -Wvolatile for all 12 combinations of TIMING_TEST_MODE × TIMING_USE_STM32_LL × TIMING_IRQ_SOURCE_TIM6, and functionally exercised with emulated DWT/TIM6/GPIO (ISR body × 3000, reporter × 3). The hardware build is done in your OwnTech project (Section 7.1).

/*

* =============================================================================

*  OwnTech SPIN / TWIST -- Critical-task timing benchmark   (v2, reviewed)

* =============================================================================

*

*  Three independent measurements of the same critical task:

*   (A) DWT_CYCCNT  : CPU-cycle counter, 1-cycle resolution (internal)

*   (B) GPIO pulse  : HIGH while the target runs, measured on an oscilloscope

*   (C) TIM6->CNT   : read at task entry = interrupt-entry latency (0.1 us)

*  plus the actual task period (jitter) and an overrun counter.

*

*  Target : STM32G474RE, Cortex-M4F @ 170 MHz (SPIN 1.2.0), OwnTech Core,

*           Zephyr RTOS, PlatformIO.

*

*  Facts verified against the OwnTech Core sources (github owntech-foundation/core):

*   - GpioHAL: configurePin/setPin/resetPin take a uint8_t. SPIN pin 9 maps to

*     GPIOC bit 7 (PC7). It is the pin used in OwnTech's own GPIO example and

*     it is not reserved by the TWIST v1.4.x shield overlay. (Do not start the

*     TIM3 incremental encoder: it would claim PC6/PC7.)

*   - TaskAPI::createCritical(fn, period_us, source_tim6): 1..6553 us. The task

*     is executed INSIDE the TIM6 UPDATE interrupt, registered with

*     IRQ_ZERO_LATENCY. It is an ISR: no printk, no k_* calls, no blocking.

*   - TaskAPI::startCritical(false) is mandatory here. With TIM6 and the

*     default argument (true), the scheduler reads the HRTIM period, which is

*     0 when PWM is not initialised, and returns WITHOUT starting the task.

*   - CPU clock: spin.dts sets rcc clock-frequency = 170 MHz; Zephyr exposes

*     it as CONFIG_SYS_CLOCK_HW_CYCLES_PER_SEC. The OwnTech TIM6 driver

*     derives its 0.1 us tick from the same constant (prescaler = f/1e7 - 1).

*   - Console: USB CDC-ACM (zephyr,console = cdc_acm_uart0), 115200 baud.

*

*  Safety: the power stage, PWM outputs and ADC acquisition are never enabled.

*

*  Quick start:

*   1. TIMING_TEST_MODE = 0  -> E0  (instrumentation overhead)

*   2. TIMING_TEST_MODE = 2  -> E0b (10 us reference: validates f_CPU on scope)

*   3. TIMING_TEST_MODE = 1  -> E1..E5, sweep TEST_SORT_N = 4, 8, 16, 32, 64

*   4. Replace task_under_test() by the MMC algorithm       -> E6

* =============================================================================

*/

#include <stdint.h>

#include <stdbool.h>

#include <zephyr/kernel.h>

#include <zephyr/sys/printk.h>

#include "SpinAPI.h"

#include "TaskAPI.h"

/* ============================ User configuration ============================ */

/* Control period Ts in microseconds. TIM6 source allows 1..6553 us. */

#define CONTROL_PERIOD_US        100U

/* 1 = TIM6 triggers the critical task (default for E0..E6, no PWM needed).

* 0 = HRTIM master timer triggers it (E7, full loop): PWM must be initialised

*     BEFORE createCritical() and Ts must be a multiple of the PWM period.

*     Entry-latency measurement (TIM6->CNT) is only meaningful with TIM6. */

#define TIMING_IRQ_SOURCE_TIM6   1

/* Oscilloscope pin. SPIN header pin number (1..58) or STM32 name (PC7...).

* Default 9 = PC7: free on TWIST, used in OwnTech's GPIO example. */

#define TIMING_GPIO_PIN          9U

/* Executions per reported batch. 1000 @ Ts=100us -> one report every 100 ms. */

#define BATCH_SAMPLES            1000U

/* Bubble-sort workload size (E1..E5 sweep: 4, 8, 16, 32, 64). */

#define MAX_SORT_N               64U

#define TEST_SORT_N              16U

/* Reference busy-wait length for TIMING_TEST_MODE 2 (clock cross-check). */

#define REFERENCE_DELAY_US       10U

/* 0 = empty window        : instrumentation calibration            (E0)

* 1 = bubble sort         : worst-case (reverse-sorted) input       (E1..E5)

* 2 = reference delay     : DWT-timed REFERENCE_DELAY_US busy-wait  (E0b)

*                           -> scope must show REFERENCE_DELAY_US + E0 offset.

*                              If not, f_CPU is not what CPU_FREQ_HZ says.   */

#define TIMING_TEST_MODE         1

/* 1 = STM32 LL register access (recommended):

*       - GPIO edges written directly to BSRR/BRR (~1 cycle instead of the

*         ~100-200 cycles of the generic Zephyr GPIO path),

*       - TIM6->CNT read at task entry = interrupt-entry latency.

* 0 = portable path: OwnTech spin.gpio API only, no latency measurement.

* If <stm32_ll_gpio.h> is not found by your build, set this to 0.           */

#define TIMING_USE_STM32_LL      1

/* 1 = also print one machine-readable "CSV,..." line per batch.             */

#define PRINT_CSV                1

/* ============================ Derived constants ============================= */

#ifdef CONFIG_SYS_CLOCK_HW_CYCLES_PER_SEC

#define CPU_FREQ_HZ  ((uint32_t)CONFIG_SYS_CLOCK_HW_CYCLES_PER_SEC)

#else

#define CPU_FREQ_HZ  170000000UL

#endif

/* TIM6 ticks at 0.1 us (OwnTech timer driver). Cycles per tick = f/10 MHz. */

#define TIM6_TICK_HZ             10000000UL

#define CYCLES_PER_TIM6_TICK     (CPU_FREQ_HZ / TIM6_TICK_HZ)

#if TEST_SORT_N > MAX_SORT_N

#error "TEST_SORT_N must be <= MAX_SORT_N"

#endif

#if (CONTROL_PERIOD_US < 1U) || (CONTROL_PERIOD_US > 6553U)

#error "CONTROL_PERIOD_US must be within 1..6553 us for the TIM6 source"

#endif

#if (TIMING_TEST_MODE < 0) || (TIMING_TEST_MODE > 2)

#error "Unsupported TIMING_TEST_MODE"

#endif

#if TIMING_IRQ_SOURCE_TIM6

#define TIMING_IRQ_SOURCE        source_tim6

#else

#define TIMING_IRQ_SOURCE        source_hrtim

#endif

/* Entry latency is read from TIM6->CNT, so it needs LL access AND TIM6 source. */

#define MEASURE_ENTRY_LATENCY    (TIMING_USE_STM32_LL && TIMING_IRQ_SOURCE_TIM6)

#if TIMING_USE_STM32_LL

#include <stm32_ll_gpio.h>

#include <stm32_ll_tim.h>

#endif

/* ======================= Cortex-M4 DWT / CoreDebug ========================== */

/* Standard Cortex-M4 addresses (ARM DDI0439, ST PM0214). No CMSIS needed.

* The #ifndef guards allow a host-side unit test to redirect the registers. */

#ifndef COREDEBUG_DEMCR

#define COREDEBUG_DEMCR   (*(volatile uint32_t *)0xE000EDFCUL)

#endif

#ifndef DWT_CTRL_REG

#define DWT_CTRL_REG      (*(volatile uint32_t *)0xE0001000UL)

#endif

#ifndef DWT_CYCCNT_REG

#define DWT_CYCCNT_REG    (*(volatile uint32_t *)0xE0001004UL)

#endif

#define COREDEBUG_TRCENA  (1UL << 24)

#define DWT_CYCCNTENA     (1UL << 0)

#define DWT_NOCYCCNT      (1UL << 25)   /* read-only: 1 = no cycle counter */

/* ============================ SPIN pin mapping ============================== */

/* Replica of GpioHAL::getPinNumber()/getGpioDevice() (OwnTech Core), so that

* the LL fast path and the start-up banner always agree with TIMING_GPIO_PIN.

* port: 0 = GPIOA, 1 = GPIOB, 2 = GPIOC, 3 = GPIOD                            */

struct spin_pin_map_t

{

uint8_t spin_pin;

uint8_t port;

uint8_t bit;

};

static constexpr spin_pin_map_t k_spin_pin_map[] =

{

{ 1, 1, 11}, { 2, 1, 12}, { 4, 1, 13}, { 5, 1, 14}, { 6, 1, 15},

{ 7, 2,  6}, { 9, 2,  7}, {10, 2,  8}, {11, 2,  9}, {12, 0,  8},

{14, 0,  9}, {15, 0, 10}, {16, 2, 10}, {17, 2, 11}, {19, 2, 12},

{20, 1,  4}, {21, 1,  9}, {22, 2, 13}, {24, 2,  0}, {25, 2,  1},

{26, 2,  2}, {27, 2,  3}, {29, 0,  0}, {30, 0,  1}, {31, 1,  0},

{32, 0,  5}, {34, 0,  6}, {35, 2,  4}, {37, 1,  1}, {41, 1, 10},

{42, 1,  2}, {43, 2,  5}, {44, 0,  7}, {45, 0,  4}, {46, 0, 13},

{47, 0, 14}, {48, 0, 15}, {49, 3,  2}, {50, 1,  3}, {51, 0,  2},

{52, 0,  3}, {53, 1,  5}, {55, 1,  6}, {56, 1,  7}, {58, 1,  8}

};

/* Returns port index (0..3) or -1. Accepts SPIN numbers and PA0..PD3 codes. */

static constexpr int spin_pin_port(uint8_t pin)

{

if ((pin & 0x80U) != 0U)

{

return (int)((pin >> 4) & 0x03U);      /* 0x8x=A 0x9x=B 0xAx=C 0xBx=D */

}

for (const spin_pin_map_t &e : k_spin_pin_map)

{

if (e.spin_pin == pin) return (int)e.port;

}

return -1;

}

static constexpr int spin_pin_bit(uint8_t pin)

{

if ((pin & 0x80U) != 0U)

{

return (int)(pin & 0x0FU);

}

for (const spin_pin_map_t &e : k_spin_pin_map)

{

if (e.spin_pin == pin) return (int)e.bit;

}

return -1;

}

static constexpr int k_timing_port = spin_pin_port((uint8_t)TIMING_GPIO_PIN);

static constexpr int k_timing_bit  = spin_pin_bit((uint8_t)TIMING_GPIO_PIN);

static_assert(k_timing_port >= 0 && k_timing_bit >= 0,

"TIMING_GPIO_PIN is not a GPIO-capable SPIN pin (see GpioHAL.cpp)");

static_assert(!(k_timing_port == 0 && (k_timing_bit == 13 || k_timing_bit == 14)),

"PA13/PA14 are SWDIO/SWCLK: do not use them as the timing pin");

#if TIMING_USE_STM32_LL

static constexpr uintptr_t k_timing_port_base =

(k_timing_port == 0) ? (uintptr_t)GPIOA_BASE :

(k_timing_port == 1) ? (uintptr_t)GPIOB_BASE :

(k_timing_port == 2) ? (uintptr_t)GPIOC_BASE : (uintptr_t)GPIOD_BASE;

#define TIMING_LL_PORT   ((GPIO_TypeDef *)k_timing_port_base)

#define TIMING_LL_MASK   (1UL << (uint32_t)k_timing_bit)

#endif

/* ============================ Timing statistics ============================= */

typedef struct

{

uint32_t min_cycles;

uint32_t max_cycles;

uint64_t sum_cycles;

uint32_t count;

} timing_accumulator_t;

typedef struct

{

uint32_t min;

uint32_t avg;

uint32_t max;

uint32_t count;

} timing_summary_t;

/* Written by the ISR, read by the background thread (seqlock on batch_id). */

typedef struct

{

volatile uint32_t batch_id;

volatile timing_summary_t target;     /* task_under_test() only        */

volatile timing_summary_t critical;   /* whole user callback body      */

volatile timing_summary_t latency;    /* TIM6 event -> callback entry  */

volatile timing_summary_t period;     /* entry-to-entry (jitter)       */

volatile uint32_t overruns;           /* body >= Ts in this batch      */

volatile uint32_t overruns_total;     /* since boot                    */

} timing_snapshot_t;

static timing_accumulator_t g_target_acc;

static timing_accumulator_t g_critical_acc;

static timing_accumulator_t g_latency_acc;

static timing_accumulator_t g_period_acc;

static uint32_t             g_overruns_batch = 0U;

static uint32_t             g_overruns_total = 0U;

static uint32_t             g_prev_start     = 0U;

static bool                 g_have_prev      = false;

static uint32_t             g_budget_cycles  = 0U;

static timing_snapshot_t    g_snapshot;

#if TIMING_TEST_MODE == 1

/* Workload data. Replace with the real MMC algorithm for E6. */

static float          g_sort_data[MAX_SORT_N];

static volatile float g_result_guard = 0.0F;

static uint32_t       g_pattern_counter = 0U;

#endif

/* ================================ Helpers =================================== */

static inline void compiler_barrier(void)

{

__asm__ volatile ("" ::: "memory");

}

static inline uint32_t dwt_now(void)

{

return DWT_CYCCNT_REG;

}

/* Returns true if the cycle counter exists and is counting. */

static bool dwt_init(void)

{

/* No compound assignment on volatile: deprecated in C++20 (-Wvolatile). */

COREDEBUG_DEMCR = COREDEBUG_DEMCR | COREDEBUG_TRCENA;

compiler_barrier();

if ((DWT_CTRL_REG & DWT_NOCYCCNT) != 0U)

{

return false;

}

DWT_CYCCNT_REG = 0U;

DWT_CTRL_REG = DWT_CTRL_REG | DWT_CYCCNTENA;

compiler_barrier();

const uint32_t a = dwt_now();

for (uint32_t i = 0U; i < 50U; ++i)

{

compiler_barrier();        /* keeps the loop, no volatile needed */

}

const uint32_t b = dwt_now();

return (b != a);

}

static void stats_reset(timing_accumulator_t *s)

{

s->min_cycles = UINT32_MAX;

s->max_cycles = 0U;

s->sum_cycles = 0U;

s->count = 0U;

}

static inline void stats_add(timing_accumulator_t *s, uint32_t cycles)

{

if (cycles < s->min_cycles) s->min_cycles = cycles;

if (cycles > s->max_cycles) s->max_cycles = cycles;

s->sum_cycles += cycles;

s->count++;

}

static void stats_publish(volatile timing_summary_t *dst,

const timing_accumulator_t *src)

{

dst->min   = (src->count != 0U) ? src->min_cycles : 0U;

dst->max   = src->max_cycles;

dst->avg   = (src->count != 0U) ? (uint32_t)(src->sum_cycles / src->count) : 0U;

dst->count = src->count;

}

static void summary_copy(timing_summary_t *dst, const volatile timing_summary_t *src)

{

dst->min   = src->min;

dst->avg   = src->avg;

dst->max   = src->max;

dst->count = src->count;

}

static uint64_t cycles_to_ns(uint32_t cycles)

{

return ((uint64_t)cycles * 1000000000ULL) / (uint64_t)CPU_FREQ_HZ;

}

/* "%u cycles (%u.%03u us)" -- avoids 64-bit printk formats. */

static void print_cycles(uint32_t cycles)

{

const uint64_t ns = cycles_to_ns(cycles);

printk("%6u cyc = %4u.%03u us",

cycles, (uint32_t)(ns / 1000ULL), (uint32_t)(ns % 1000ULL));

}

static void print_summary(const char *label, const timing_summary_t *s)

{

printk("  %-14s min ", label); print_cycles(s->min);

printk("   avg ");            print_cycles(s->avg);

printk("   max ");            print_cycles(s->max);

printk("\n");

}

/* ============================ GPIO edge helpers ============================= */

static inline void timing_pin_high(void)

{

#if TIMING_USE_STM32_LL

LL_GPIO_SetOutputPin(TIMING_LL_PORT, TIMING_LL_MASK);

#else

spin.gpio.setPin((uint8_t)TIMING_GPIO_PIN);

#endif

}

static inline void timing_pin_low(void)

{

#if TIMING_USE_STM32_LL

LL_GPIO_ResetOutputPin(TIMING_LL_PORT, TIMING_LL_MASK);

#else

spin.gpio.resetPin((uint8_t)TIMING_GPIO_PIN);

#endif

}

/* ============================ Benchmark workloads =========================== */

#if TIMING_TEST_MODE == 1

static void prepare_sort_input(void)

{

/* Reverse-sorted input = worst case for bubble sort (N(N-1)/2 compares

* and swaps, early-exit never taken). Runs OUTSIDE the target window.

* The small varying offset defeats any cross-iteration constant folding. */

const float offset = (float)(g_pattern_counter & 0x7U) * 0.001F;

for (uint32_t i = 0U; i < TEST_SORT_N; ++i)

{

g_sort_data[i] = (float)(TEST_SORT_N - i) + offset;

}

g_pattern_counter++;

}

__attribute__((noinline)) static void bubble_sort_task(void)

{

for (uint32_t i = 0U; i + 1U < TEST_SORT_N; ++i)

{

bool swapped = false;

for (uint32_t j = 0U; j + 1U < TEST_SORT_N - i; ++j)

{

if (g_sort_data[j] > g_sort_data[j + 1U])

{

const float tmp     = g_sort_data[j];

g_sort_data[j]      = g_sort_data[j + 1U];

g_sort_data[j + 1U] = tmp;

swapped = true;

}

}

if (!swapped) break;

}

}

#endif /* TIMING_TEST_MODE == 1 */

#if TIMING_TEST_MODE == 2

/* Busy-waits exactly REFERENCE_DELAY_US according to DWT. The oscilloscope

* must then show REFERENCE_DELAY_US + (E0 pulse overhead). A systematic

* deviation means CPU_FREQ_HZ does not match the real core clock.          */

__attribute__((noinline)) static void reference_delay_task(void)

{

const uint32_t start = dwt_now();

const uint32_t wait  = REFERENCE_DELAY_US * (CPU_FREQ_HZ / 1000000UL);

while ((dwt_now() - start) < wait)

{

compiler_barrier();

}

}

#endif /* TIMING_TEST_MODE == 2 */

__attribute__((noinline)) static void task_under_test(void)

{

#if TIMING_TEST_MODE == 0

compiler_barrier();            /* empty: instrumentation overhead only */

#elif TIMING_TEST_MODE == 1

bubble_sort_task();            /* <- replace by the MMC algorithm (E6) */

#else

reference_delay_task();

#endif

}

/* ============================= Snapshot (ISR side) ========================== */

static void publish_snapshot_if_ready(void)

{

if (g_target_acc.count < BATCH_SAMPLES) return;

stats_publish(&g_snapshot.target,   &g_target_acc);

stats_publish(&g_snapshot.critical, &g_critical_acc);

stats_publish(&g_snapshot.latency,  &g_latency_acc);

stats_publish(&g_snapshot.period,   &g_period_acc);

g_snapshot.overruns       = g_overruns_batch;

g_snapshot.overruns_total = g_overruns_total;

/* All fields complete before the id changes. The writer is an ISR that the

* reader thread can never interrupt, so a single counter is sufficient. */

compiler_barrier();

g_snapshot.batch_id = g_snapshot.batch_id + 1U;

stats_reset(&g_target_acc);

stats_reset(&g_critical_acc);

stats_reset(&g_latency_acc);

stats_reset(&g_period_acc);

g_overruns_batch = 0U;

}

/* ====================== Critical task (TIM6 ISR context) ==================== */

static void loop_critical_task(void)

{

#if MEASURE_ENTRY_LATENCY

/* TIM6 counts 0.1 us ticks from 0 at each UPDATE event: CNT here is the

* time from the timer event to the first instruction of this function

* (NVIC entry + Zephyr ISR wrapper + OwnTech proxy). */

const uint32_t entry_ticks = LL_TIM_GetCounter(TIM6);

#endif

const uint32_t critical_start = dwt_now();

#if TIMING_TEST_MODE == 1

prepare_sort_input();

#endif

/* ---- measured target: GPIO HIGH ... LOW brackets the DWT window ---- */

timing_pin_high();

compiler_barrier();

const uint32_t target_start = dwt_now();

task_under_test();

compiler_barrier();

const uint32_t target_end = dwt_now();

timing_pin_low();

/* -------------------------------------------------------------------- */

#if TIMING_TEST_MODE == 1

/* Visible side effect so the optimiser cannot discard the sort. */

g_result_guard = g_sort_data[0] + g_sort_data[TEST_SORT_N - 1U];

#endif

const uint32_t critical_end = dwt_now();

/* Unsigned differences are wrap-safe for intervals < 2^32 cycles (25 s). */

const uint32_t target_cycles   = target_end   - target_start;

const uint32_t critical_cycles = critical_end - critical_start;

stats_add(&g_target_acc,   target_cycles);

stats_add(&g_critical_acc, critical_cycles);

#if MEASURE_ENTRY_LATENCY

stats_add(&g_latency_acc, entry_ticks * (uint32_t)CYCLES_PER_TIM6_TICK);

#endif

if (g_have_prev)

{

stats_add(&g_period_acc, critical_start - g_prev_start);

}

g_prev_start = critical_start;

g_have_prev  = true;

if (critical_cycles >= g_budget_cycles)

{

g_overruns_batch++;

g_overruns_total++;

}

publish_snapshot_if_ready();

}

/* ========================= Background reporting task ======================== */

static void loop_background_task(void)

{

static uint32_t last_batch_id = 0U;

uint32_t id_before;

uint32_t id_after;

timing_summary_t t, c, l, p;

uint32_t overruns, overruns_total;

do

{

id_before = g_snapshot.batch_id;

compiler_barrier();

summary_copy(&t, &g_snapshot.target);

summary_copy(&c, &g_snapshot.critical);

summary_copy(&l, &g_snapshot.latency);

summary_copy(&p, &g_snapshot.period);

overruns       = g_snapshot.overruns;

overruns_total = g_snapshot.overruns_total;

compiler_barrier();

id_after = g_snapshot.batch_id;

}

while (id_before != id_after);

if ((id_before != 0U) && (id_before != last_batch_id) && (t.count != 0U))

{

last_batch_id = id_before;

const uint32_t budget = g_budget_cycles;

/* Worst case seen by the CPU = entry latency + body (exit not included). */

const uint32_t worst_isr = l.max + c.max;

printk("\n=== Timing batch %u ===\n", id_before);

#if TIMING_TEST_MODE == 0

printk("Mode: 0 EMPTY CALIBRATION");

#elif TIMING_TEST_MODE == 1

printk("Mode: 1 BUBBLE SORT, N=%u", (uint32_t)TEST_SORT_N);

#else

printk("Mode: 2 REFERENCE DELAY, %u us", (uint32_t)REFERENCE_DELAY_US);

#endif

printk(" | Ts=%u us | f_CPU=%u Hz | pin %u (P%c%d) | LL=%d\n",

(uint32_t)CONTROL_PERIOD_US, (uint32_t)CPU_FREQ_HZ,

(uint32_t)TIMING_GPIO_PIN, (char)('A' + k_timing_port),

k_timing_bit, (int)TIMING_USE_STM32_LL);

printk("Samples: %u\n", t.count);

print_summary("Target",        &t);

print_summary("Critical body", &c);

#if MEASURE_ENTRY_LATENCY

print_summary("Entry latency", &l);

#else

printk("  Entry latency  n/a (needs TIMING_USE_STM32_LL=1 and TIM6 source)\n");

#endif

print_summary("Period",        &p);

printk("  Period jitter  (max-min): "); print_cycles(p.max - p.min); printk("\n");

printk("  Period budget  (Ts)     : "); print_cycles(budget);        printk("\n");

if ((overruns == 0U) && (worst_isr < budget))

{

const uint32_t slack    = budget - worst_isr;

const uint32_t util_x10 = (uint32_t)(((uint64_t)worst_isr * 1000ULL) / budget);

printk("Deadline: PASS\n");

printk("  Worst case (latency+body): "); print_cycles(worst_isr); printk("\n");

printk("  Worst-case slack         : "); print_cycles(slack);     printk("\n");

printk("  Worst-case utilisation   : %u.%u %%\n",

util_x10 / 10U, util_x10 % 10U);

}

else

{

printk("Deadline: FAIL  (overruns in batch: %u, total: %u)\n",

overruns, overruns_total);

}

printk("Scope: HIGH-pulse width on pin %u = Target + E0 offset\n",

(uint32_t)TIMING_GPIO_PIN);

#if PRINT_CSV

/* CSV,batch,mode,N,samples,t_min,t_avg,t_max,c_min,c_avg,c_max,

*     l_min,l_avg,l_max,p_min,p_avg,p_max,overruns,budget   (cycles) */

printk("CSV,%u,%d,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u,%u\n",

id_before, (int)TIMING_TEST_MODE, (uint32_t)TEST_SORT_N, t.count,

t.min, t.avg, t.max, c.min, c.avg, c.max,

l.min, l.avg, l.max, p.min, p.avg, p.max, overruns, budget);

#endif

}

task.suspendBackgroundMs(100U);

}

/* ============================== Setup and main ============================== */

static void setup_routine(void)

{

const bool dwt_ok = dwt_init();

/* Pin configured through the OwnTech API (enables the port clock and sets

* the mode). The LL fast path then only writes BSRR/BRR of that pin. */

spin.gpio.configurePin((uint8_t)TIMING_GPIO_PIN, OUTPUT);

spin.gpio.resetPin((uint8_t)TIMING_GPIO_PIN);

stats_reset(&g_target_acc);

stats_reset(&g_critical_acc);

stats_reset(&g_latency_acc);

stats_reset(&g_period_acc);

g_snapshot.batch_id = 0U;

g_budget_cycles = (uint32_t)(((uint64_t)CPU_FREQ_HZ * CONTROL_PERIOD_US) / 1000000ULL);

const int8_t background_task = task.createBackground(loop_background_task);

const int8_t critical_status =

task.createCritical(loop_critical_task, CONTROL_PERIOD_US, TIMING_IRQ_SOURCE);

printk("\nOwnTech SPIN/TWIST timing benchmark v2\n");

printk("  CPU clock (CONFIG_SYS_CLOCK_HW_CYCLES_PER_SEC): %u Hz\n", (uint32_t)CPU_FREQ_HZ);

printk("  Control period Ts: %u us = %u cycles\n", (uint32_t)CONTROL_PERIOD_US, g_budget_cycles);

printk("  Timing pin: SPIN %u -> P%c%d, LL fast path: %d\n",

(uint32_t)TIMING_GPIO_PIN, (char)('A' + k_timing_port), k_timing_bit,

(int)TIMING_USE_STM32_LL);

printk("  Mode: %d, N: %u, batch: %u samples, IRQ source: %s\n",

(int)TIMING_TEST_MODE, (uint32_t)TEST_SORT_N, (uint32_t)BATCH_SAMPLES,

TIMING_IRQ_SOURCE_TIM6 ? "TIM6" : "HRTIM");

printk("  DWT cycle counter: %s\n", dwt_ok ? "OK" : "NOT AVAILABLE");

#if PRINT_CSV

printk("CSV_HEADER,batch,mode,N,samples,t_min,t_avg,t_max,c_min,c_avg,c_max,"

"l_min,l_avg,l_max,p_min,p_avg,p_max,overruns,budget_cycles\n");

#endif

if (background_task >= 0)

{

task.startBackground((uint8_t)background_task);

}

else

{

printk("ERROR: could not create background task.\n");

}

if ((critical_status == 0) && dwt_ok)

{

/* false is REQUIRED: with TIM6 and no PWM, the default (true) makes

* startCritical() return early without starting the task. */

task.startCritical(false);

}

else

{

printk("ERROR: critical task not started (create=%d, dwt=%d).\n",

(int)critical_status, (int)dwt_ok);

}

}

int main(void)

{

setup_routine();

return 0;

}

# 11. References

- OwnTech Core sources (verified): github.com/owntech-foundation/core — zephyr/modules/owntech_spin_api/zephyr/src/GpioHAL.h, GpioHAL.cpp; zephyr/modules/owntech_task_api/zephyr/public_api/TaskAPI.h; zephyr/modules/owntech_task_api/zephyr/src/uninterruptible_synchronous_task.cpp; zephyr/modules/owntech_timer_driver/zephyr/src/stm32_timer_driver.c; zephyr/modules/owntech_hrtim_driver/zephyr/src/hrtim.c; zephyr/boards/owntech/spin/spin.dts; zephyr/dts/hrtim.dtsi; zephyr/dts/bindings/power-leg.yaml; zephyr/boards/shields/twist/twist_v1_4_2.overlay; zephyr/prj.conf; platformio.ini.

- OwnTech documentation — Task API: https://docs.owntech.org/latest/core/docs/task_introduction/ ; GPIO API: https://docs.owntech.org/latest/core/docs/spin_gpio/ ; SPIN 1.2.0 getting started and pinout figure: https://docs.owntech.org/latest/spin/1.2.0/getting_started/

- ARM Cortex-M4 Technical Reference Manual (DDI0439), DWT chapter; ST PM0214 (Cortex-M4 programming manual for STM32); ST RM0440 (STM32G4 reference manual), GPIO BSRR/BRR and TIM6 chapters.

- Zephyr RTOS documentation — Zero-latency interrupts; cbprintf/printk; Kconfig CONFIG_SIZE_OPTIMIZATIONS / CONFIG_SPEED_OPTIMIZATIONS.